Parking Lock Installation and Operation (Brief instruction)

- Position: It is advisable to place and install the Parking Lock in the center of the parking lot.

- The unit should be installed on plane and dry surface, for example blacktop. Take the key (included in the delivery), unlock and open the upper cover of the drive housing and lift the cover. Fix the lower part of the drive housing with 3 bolts (Included in the delivery) into the ground by using the designated spots.

- Before initial setup, charge the battery for at least 10 hours. Take the battery out of the holder and detach (turning off) the connecting cable from the drive port. Connect the battery cable with the power adapter (included in the delivery) and plug it into a power outlet. After the battery is fully charged insert the battery back into the designated holder. Put the upper cover back on the drive unit and lock the cover again. Attention! The Parking Lock only works properly in the locked position. Remove the key and your Parking Lock is ready for use.

- By pushing the upper two buttons of the remote control, the barrier bail can be placed up and down. The operation of the barrier bail is also indicated by the indicator light, which is located in the front. The range of the infrared remote control averages 5 meter. The Parking Lock can be operated conveniently out of the car.

- In case of an unauthorized try of disengagement, by pulling down the barrier bail, alarm sounds for a short time and the barrier bail goes up automatically which insures that no unauthorized person can use your parking spot.

- The durability of the battery averages about 3 months, depending of the frequency of use. After that, the battery has to be recharged for another 10 hours.